| Номер |  | Описание | Кол-во |
| --- | --- | --- | --- |
| 30 | 17377 | SKET-TRUSIOMA Экструдер 160 мм 25 D | 1 |
| 31 | 17396 | NOKIA Двухслойная экструзионная головка, Модель PFE 4/6 | 1 |
| 31 | 17397 | PLASTICOLOR Система дозирования с двумя дозирующими частями, Модель 2000 | 2 |
| 31 | 17320 | BETA Измеритель емкости и KI 1700 контроллер | 1 |
| 41 | 17331 | NIEHAUS Приемное устройство валового типа 1000 мм, с фрикционным зажимом | 1 |
| 41 | 17349 | Консольное приемное устройство 1000 мм валового типа | 1 |
| 42 | 17378 | SKALTEK Приводное отдающее устройство, МодельA16-4K | 1 |
| 42 | 17332 | POURTIER 5 x 500 мм Приводное отдающее устройство | 1 |
| 43 | 17304 | Бухтовочная головка 600 mm | 1 |
| 43 | 17305 | Горизонтальная бухтовочная головка 500 мм | 1 |
| 45 | 17330 | NIEHAUS Одноколесное тяговое устройство 1000 мм | 1 |
| 50 | 17333 | POURTIER Устройство натяжения ременного типа | 10 |
| 50 | 17334 | POURTIER Устройство предварительной подкрутки с 3-мя блоками | 1 |
| 90 | 17317 | Стальной барабан 1600 X 800 X 80 (фланец X наружная ширина X отверстие мм) | 106 |
| 90 | 17318 | Стальная катушка 630 X 400 X 127 (фланец X наружная ширина X отверстие мм) | 13 |
| 90 | 17328 | Стальной барабан 2200 X 1100 X 100 (фланец X наружная ширина X отверстие мм) | 25 |
| 90 | 17395 | Стальная катушка 900 X 550 X 80 (фланец X наружная ширина X отверстие мм) | 24 |

1 X SKET-TRUSIOMA Heavy Duty 160mm 25 D Extruder (our ref. 30– 17377)

Stand alone strong extruder designed to process PVC, PE with 2 dedicated screws. Both screws are bored for thermoregulation.

| Extruder type Nbr of zones Cooling Screws Drive Transission Screw speed Drive | 160 mm - 25D 8 8 fans AC motor 2, barrier type DC motor 255 kW/1535 rpm Belts (5) + reducer 60 rpm SSD 590+ DC in line cabinet |
| --- | --- |

1 X NOKIA Dual Layer Extrusion Head,  Model PFE 4/6 (our ref. 31– 17396)

Complete extrusiion head previously used in a telephone wire extrusion line. Delivery includes: - Full head support - Extrusion head - Connection flange for Nokia Maillefer 60 mm extruder - Inlet for 30 mm piggy back extruder - Breaker plate - Heating zones

| Max. diameter in Max diameter out Original line direction | 4 mm 6 mm L > R |
| --- | --- |

2 X PLASTICOLOR Dosing System with 2 Dosing Units, Model 2000 (our ref. 31– 17397)

Complete set of 2 Plasticolor 2000 mounted on a common feeding neck. Unit is complete with operator panel. Neck is adapted to Nokia Maillefer 60 mm extruder

| Type Hopper capacity | PC 2000 +- 20 L |
| --- | --- |

1 X BETA Capacity gauge and KI 1700 Controller (our ref. 31– 17320)

Complete set for the measuring of cable capacitance in telephone and data field. Composition: - Immersed gauge - Controller unit KI 1700 in 19» rack

1 X NIEHAUS Shaft Type Take-up 1000 mm, with Friction Clutch (our ref. 41– 17331)

Heavy duty shaft type take-up well adapted to be used in tubular strander, closing machine or similar heavy process. The bobbin is taken on shaft. It is driven from main machine by chain. Winding tension is preset by mechanical friction clutch. Complete with traversing. Was used before with a 8 B tubular strander 400 mm

| Take Up dia. Overall width Drum holding Drum Lifting Drive Traversing | 1000 mm 750 mm Shaft Manual by chain from capstan by screw/cam |
| --- | --- |

1 X Home Made Cantilever 1000 mm Shaft type Take Up (our ref. 41– 17349)

Home Made 1000mm Cantilever Take Up

| Bobbin dimension Bobbin holding Traversing Lifting | 1000mm Shaft type Uhing AC motor + Clutch |
| --- | --- |

1 X SKALTEK Driven Pay Off, Model A16-4K (our ref. 42– 17378)

Portal design allows for compact and stable reel handling. Roll through design and self locating pintles makes loading and unloading fast and easy. Pintle is driven by DC motor with reducer (no gear box) Can be equiped with a traversing unit in order to be used as a Take up

| Reel diameter Reel width Reel weight Bobbin held Loading/unloading Bobbin size set-up | 750 - 1650 mm 600 - 1060 mm 2500 kg between pintles Electric Electric |
| --- | --- |

1 X POURTIER 5 x 500 mm Unrolling & Driven Pay off Stand (our ref. 42– 17332)

This stand is made to pay-off up to 5 conductors or pairs from DIN 500 bobbins in a driven way. It includes «dancing stand» with 5 dancer for fine tension control. Bobbins are in-line and hold between pneumatic pintles. Each bobbin is driven by own AC motor. Loading is made from the top. Electrical panel is not complete.

| Bobbins qty Diameter OAW Holding Pintles in/out Loading Motor Tension control Electrical panel Line direction | 5 500 mm 375 mm Btw pintles Pneumatic From top AC, 1.6 A, 175 V, 750 rpm 50 Hz Weighted dancer Not complete Right to Left |
| --- | --- |

1 X Stand alone 600 mm Coiling Head (our ref. 43– 17304)

Complete stand alone Coiling Head made of a strong steel frame equipped with four castor wheels and four blocking pads. Collapsible head is driven by an AC motor through pulleys and belts. Operator has a foot pedal to control coilng operation There is no traversing system.

| Coil OD Coil ID Coil width Drive Control | 600 mm 300 mm 300 mm AC 2 kW / 690 rpm Foot switch |
| --- | --- |

1 X Stand alone 500 mm Horizontal coiling head (our ref. 43– 17305)

Complete stand alone strong design coiling head. Head is fitted with removable outer flange of 500 mm diameter. Coiling speed can be adjusted by operator thanks to electrically controlled reducer. Ther is no traversing system. Assembly is fixed onto a large and stable base plate. Operator controls the unit through footswitch and speed control switch.

| Outer flange diameter Inner flange diameter Core shape / diameter Coiling width Drive Reducer final speed range Speed control | 500 mm 600 mm Conical / 180 to 240 mm 150 mm AC 1,10 kW / 900 rpm / 380 V Brottby 45 - 290 rpm Spring loaded switch |
| --- | --- |

1 X NIEHAUS Heavy Duty Single Wheel Capstan 1000 mm (our ref. 45– 17330)

Heavy duty pulling capstan 1000 mm equipped with gear box which can be used in starnding line like tubular or equivalent. The unit includes wheel as well as heavy duty reducer/gear box. The machine was installed previously in a tubular line 8 x 400 mm used for steel stranding/closing operation.

| Capstan Diameter Lay Length Gearbox Drive Direction | Single wheel 1000 mm 17,2 to 90,2 mm 24 steps Line shaft Right to left |
| --- | --- |

10 X POURTIER Belt Type Tensioning Unit (our ref. 50– 17333)

These units were used together with basket coiler to apply a right tension over insulated wire. The conductor.cable is belt wrapped on 90° around a brake capstan. Tension is control remote from common pneumatic control desk for the 10 positions.

| Pulley diameter Groove Groove width Wrap angle Tension setting Control desk | 300 mm Flat 25 mm 90° Pneumatic 0-10 bar Available Commun for the 10 units |
| --- | --- |

1 X POURTIER 3 Pulleys ‘Pre-Twister’ (our ref. 50– 17334)

Device used for pre-twisting insulated cores in front of twisting unit. It is mechanically complete, including motor.

| Deviation pulley dia. Line height Max. cable diameter Speed Motor Electrical panel | 3*180 mm 1100 mm +/- 15 mm 500 RPM (kinematic of today) Siemens Not available |
| --- | --- |

106 X Steel Bobbin 1600 X 800 X 80 (fl X O.w. X Bore In mm) (our ref. 90– 17317)

Good capacity steel drums for general purpose in cable factories

| Flange diameter OAW Useful width Bore Barrel diameter Material | 1600 mm 800 mm 680 mm 80 mm 750 mm Steel |
| --- | --- |

13 X Steel Bobbin 630 X 400 X 127 (fl X O.w. X Bore In mm)  (our ref. 90– 17318)

Good quality 630 mm steel bobbin. This bobbin is not balanced type and is to be used in slow processes such as bunching, pairing, extrusion lines,...

| Flange diameter OAW Useful width Barrel diameter Bore Material | 630 mm 400 mm 375 mm 280 mm 127 mm Steel |
| --- | --- |

25 X   Steel Bobbin 2200 X 1100 X 100 (fl X O.w. X Bore In mm) (our ref. 90– 17328)

Strong design steel bobbins coming in two comparable dimensions

| Bobbin type 1 diameter OAW Barrel diameter Bore Qty Bobbin type 2 diameter OAW Barrel diameter Bore Qty | 2200 mm 1100 mm 990 mm 100 mm 19 2175 mm 1085 mm 1100 mm 100 mm 6 |
| --- | --- |

24 X Steel Bobbin 900 X 550 X 80 (fl X O.w. X Bore In mm) (our ref. 90– 17395)