| Reference |  | Description | Qty |
| --- | --- | --- | --- |
| 7 | 18308 | FAG Induction Heater for bearings, Model AWG8 | 1 |
| 13 | 17115 | MAILLEFER 630 Quad. Line with 4 Backtwist Pay-offs, Model TC | 1 |
| 17 | 18305 | BRONDEL 2 x 500 mm Taping Head, with PIV | 1 |
| 30 | 18299 | ITAL 30 mm Extruder | 1 |
| 30 | 18302 | MAILLEFER FEP Extruder, Model BM 45-20D | 1 |
| 41 | 18307 | SKET 2000 mm Cantilever Type Take-Up | 1 |
| 42 | 18306 | SKET 1250 mm Cantilever Pay-Off, Brake type | 1 |
| 45 | 18300 | ITAL 2 x 500 mm Capstan Model CPT500 | 1 |
| 45 | 18301 | ITAL Single Wheel 800 mm Capstan, Model CPT800 | 1 |
| 50 | 18309 | NOVA 7 wires tensioning unit | 1 |
| 86 | 18291 | IDEAL Butt Welder for Cu Model LSF 004 | 1 |
| 86 | 18292 | IDEAL Butt Welder for Cu Wires Model LS10 | 2 |
| 215 | 18294 | BWE Conform/Conklad 350 Continuous Extrusion Line | 1 |
| 310 | 18289 | BÜHLER 3 Stand Rolling Mill, with Turk Head | 1 |

1 X FAG Induction Heater for bearings, Model AWG8 (our ref. 7– 18308)

Trolley mounted induction heater regularly used for bearings preheat. Unit needs a new T° probe to be operational.

| Power T° Arm dimensions (L x W x H) | 8 kVA 120°C maximum 210 x 80 x 70 mm |
| --- | --- |

1 X MAILLEFER 630 Quad. Line with 4 Backtwist Pay-offs, Model TC (our ref. 13– 17115)

Complete line with backtwist pay-offs designed for production of high quality quads at a maximum practical speed of 4.000 tw/min. . Practically well adapted for bigger wire sizes like 1,2 mm copper. Lay length is selected electronically as take-up bobbin is driven by DC motor. Backtwist rate is also selected by potentiometer and synchronized with pay-off speed. All flyer type pay-offs are driven by one common DC motor. Loading of the bobbin is made manually from top while full bobbin is ejected from table located below. Delivery includes : - 4 backtwist payoffs 500 mm - 1 quadding machine 630 mm - 1 electrical panel and operator control

| Production speed Line speed Wire sizes Insulated cores Cable dia. Bobbin sizes on pay-offs Bobbin sizes on take-up Min Barrel size Main drive Bobbin drive Traversing system Lay-length control Backtwist control | 4,000 tw/min 500 m/min 0,3-1,2 mm 2,5 mm max 6 mm max. 450 mm (can be 500 mm) 500 mm (630 mm possible) 160 mm dia. DC motor Axem DC motor UHING system Electronic, by potentiometer Electronic, by potentiometer |
| --- | --- |

1 X BRONDEL 2 x 500 mm Taping Head, with PIV (our ref. 17– 18305)

This taping head is designed to apply tangentially two taped on cables with diameter up to +/- 90 mm maximum. Head is equipped for two pads with max dia. of 500 mm. Unit is driven from line shaft and controlled with PIV system. It can be used for paper, terphan, PE, PVC, light Cu & Alu foils. Head was formerly used in a 1+24+24 planetary line.

| Number of pads Pad OD Bore Width Speed Drive Direction Cage brake Passage | 2 500 mm 70 mm 80 mm 86 to 517 rpm Line shaft & PIV Left to right Shoe brake 110 mm |
| --- | --- |

1 X ITAL 30 mm Extruder (our ref. 30– 18299)

Nice compact 30 mm extruder Crosshead available

| Screw dia Screw length Screw drive Cooling zones | 30 mm 22.5 kW, Siemens, DC drive 3 |
| --- | --- |

1 X MAILLEFER FEP Extruder, Model BM 45-20D (our ref. 30– 18302)

Maillefer 45 mm extruder previously used in a FEP extrusion line Equipment includes: - Extuder with barrel, reducer and motor - Single Flight screw - Hopper - Extrusion head for small size wires Electrical cabinet missing, to be made new

| Barrel diameter L/D Nbr of heating zone Barrel cooling Breech zone cooling Screw type Drive Electrical cabinet | 45 mm 20 3 Fan (4) Water Single flight DC 15 kW / 3000 RPM No |
| --- | --- |

1 X SKET 2000 mm Cantilever Type Take-Up  (our ref. 41– 18307)

Cantilever take-up suitable for bobbins up to 2000 mm dia. Unit was driven from line shaft and need new motor to run as separate unit (not part of delivery).

| Take-up type Bobbin diameter Traversing Drive / Panel Bobbin holding Bobbin lifting | Cantilever 2000 mm Uhing Not available Between pintles Jacks |
| --- | --- |

1 X SKET 1250 mm Cantilever Pay-Off, Brake type (our ref. 42– 18306)

Cantilever pay-off equipped with braking system can be used in line with planetary, rigid or any kind of low speed lines. Pay-off can work with bobbins ranging 630 to 1250 mm in diameter and width from 475 to 950 mm. Bobbins is taken between pintles. Loading is achieved from ground with jack system manually actuated. Unit was formerly running in a 24 planetary stranding line.

| Pay-off type Bobbin diameter OAW Bore diameter Maximum load Back tension Bobbin holding Bobbin lifting | Cantilever 630 to 1250 mm 475 to 950 mm 80 mm 3500 kg Disc brake Between pintles Jack (manual) |
| --- | --- |

1 X ITAL 2 x 500 mm Capstan  Model CPT500 (our ref. 45– 18300)

Excellent condition double wheel capstan very well adapted for pulling sensitive cables like FOC, Coaxial... One wheel is single block driven, the other is a single one free wheeling on shaft. Drive is made by servomotor and load cell is available on shaft for sensing cable tension.

| Number of wheels Driven wheel dia Free wheel dia | 2 500 mm, 1 groove 1 pulley of 500 mm |
| --- | --- |

1 X ITAL Single Wheel 800 mm Capstan,  Model CPT800 (our ref. 45– 18301)

Excellent condition single wheel capstan very well adapted for pulling sensitive cables like FOC, Coaxial...

| Number of wheels Driven wheel dia | 1 800 mm |
| --- | --- |

1 X NOVA  7 wires tensioning unit (our ref. 50– 18309)

Stand alone tensionning unit designed to pass 7 wires in a bunching line for example. Tension equalizing is ensured by a 180 mm diameter drum controlled by an electromagnetic brake. Power control on brake by installed power supply and potentiometer. Some ceramic eyelets are missing.

| Nbr of wires Ceramic eyelet inner diameter Drum dimensions (dia x width) Power supply | 7 9 mm 180 x 160 mm Available |
| --- | --- |

1 X IDEAL Butt Welder for Cu  Model LSF 004 (our ref. 86– 18291)

Butt welding machine which can be used for welding Cu wires together or strands/bunches. When strands/bunches, it is wleded under ceramic tubes for proper diameter control. The 2 electrodes can be brought one against the other gradually. It can cover range of 0,20 mm² to 4,00 mm²,

| Transformer output Cu wire Weight Packing size | 0,6 kVA 0,2 - 4 sqmm 22 kg 380 mm X 420 mm X 380 mm |
| --- | --- |

2 X IDEAL Butt Welder for Cu Wires Model LS10 (our ref. 86– 18292)

Unit designed for welding solid wire or strands in extrusion line or others.

| Model Power supply Sections Stroke | LS 10 220 V / 18 Amps 0,25 mm² 0,50 mm² 0,75 mm² 1,00 mm² 1,50 mm² 2,50 mm² 4,00 mm² 7-20 mm |
| --- | --- |

1 X BWE  Conform/Conklad 350 Continuous Extrusion Line (our ref. 215– 18294)

Complete BWE Conform/Conklad line, model 350. Line was used before mainly for Al sheathing for CATV line. Conform process can be used for production for Al round tube, Al solid or hollow sections, small Cu buss bars, Cu sections shaped Cu conductors Conklad process can be used for Al cladding and sheathing . The line is complete and includes : 1) 4 alminium rod payoffs 2) 2 alminium rod straightener 3) 1 core cable payoff 4) 1 pulling device for core cable 5) 1 dancer for core paying off 6) 1 pinch roll for alminium rod 7) 1 Conklad machine 8) 1 Cooling trough 9)1 Dancer for tube drawing station 10) 1 Inline tube drawing station 11) 1 Capstan 12) 1 Tube cutting unit 13) 1 Dancer for takeup 14) 1 Takeujp 15) 1 Electrical and control panel

| Conform Conklad machine Output Al tube size Al tube thickness Line direction Line length Al rod pay-off Coil size Coil weight Rod diameter Central cable Pay-off Bobbin size Bobbin weight Bobbin rotation speed Drive Pulling device for core pay-off Pulling force Drive Conklad machine Wheel diameter Line speed Wheel rotation speed Product diameter Drive Cooling trough Length Pulling capstan Wheel diameter Number of turns Pulling force Drive Take-up Bobbin size Bobbin weight | 350/200 450 kg 28,5 mm 0,5 – 1,2 mm From left to right 46 m Horizontal 1400 mm 2500 kg 8 – 15 mm Cantilever type 1700 mm dia X 870 mm OAW 3000 kg 95 RPM DC 12 kW 10 kg Servo motor, 400 W 350/200 type 350 mm 120 m/min max 4 – 18 RPM 5 – 20 mm DC 185 kW 4 m Single wheel type 1000 mm 6 800 kg 45,4 kW Portal type 2200 mm dia X 1170 mm OAW 5000 kg |
| --- | --- |

1 X BÜHLER 3 Stand Rolling Mill, with Turk Head (our ref. 310– 18289)

This rolling mill is a 3 stand design with turk head as intermediate stand. It is designed for rolling Cu and Cu alloy from maximum 3.5 mm diameter to flats of thicknesses between 0.1 & 0.5 mm and width from 1.3 to 6 mm. Line is complete and composed of: - 1x Pay-off - 1x 3 Stand rolling mill - 1x Dancer - 1x Take-up - 1x Electrical panel - 1x Hanging control panel

| Output range Thickness Width Maximum input Rod diameter Max speed Number of stands Stand #1 Roll dia. Motor power Stand #2 Roll dia. Motor power Turk head Roll dia. Motor power Take-up & Pay-off Line direction | 0.1 to 0.5 mm 1.3 to 6.0 mm 3.5 mm 600 m/min 3 125 mm +/- 25 kW 125 mm +/- 25 kW 110 mm +/- 7 kW 250 kg Left to Right |
| --- | --- |